МИНИСТЕРСТВО ОБРАЗОВАНИЯ И НАУКИ РОССИЙСКОЙ ФЕДЕРАЦИИ

ФЕДЕРАЛЬНОЕ ГОСУДАРСТВЕННОЕ БЮДЖЕТНОЕ ОБРАЗОВАТЕЛЬНОЕ УЧРЕЖДЕНИЕ ВЫСШЕГО	ОБРАЗОВАНИЯ

«ВОРОНЕЖСКИЙ ГОСУДАРСТВЕННЫЙ ТЕХНИЧЕСКИЙ УНИВЕРСИТЕТ»

(ФГ БОУ ВО «ВГТУ», ВГТУ)

Факультет информационных технологий и компьютерной безопасности

(факультет)

| Кафедра | Систем информационной безопасности |
| --- | --- |

Отчёт по лабораторной работе №13

| по дисциплине |  | Сети и системы передачи информации |
| --- | --- | --- |
| Тема: | Статическая маршрутизация |  |

Расчетно-пояснительная записка

| Разработал студент: | Д. А. Коростелев |
| --- | --- |

Подпись, дата	Инициалы, фамилия

| Руководитель | доц. С.С. Куликов |
| --- | --- |

Подпись, дата	Инициалы, фамилия

Подпись, дата	Инициалы, фамилия Защищена	Оценка		_

Дата

2022

Информация об устройствах

| Название устройства | Тип и номер интерфейса | MAC-адрес интерфейса | IP-адрес интерфейса |
| --- | --- | --- | --- |
| Router0 | Gig0/0 | 0060.47DE.1201 | 192.168.140.1 |
| Router0 | Gig0/1 | 0060.47DE.1202 | 192.168.137.1 |
| Router1 | Gig0/0 | 0090.0CEC.BB01 | 192.168.140.2 |
| Router1 | Gig0/1 | 0090.0CEC.BB02 | 192.168.138.1 |
| Router1 | Gig0/2 | 0090.0CEC.BB03 | 192.168.141.1 |
| Router2 | Gig0/0 | 00D0.9739.BD01 | 192.168.141.2 |
| Router2 | Gig0/1 | 00D0.9739.BD02 | 192.168.139.1 |
| Switch0 | Gig0/1 | 0060.2F6C.5619 | - |
| Switch0 | Fa0/1 | 0060.2F6C.5601 | - |
| Switch0 | Fa0/2 | 0060.2F6C.5602 | - |
| Switch1 | Gig0/1 | 00E0.F705.6119 | - |
| Switch1 | Fa0/1 | 00E0.F705.6101 | - |
| Switch1 | Fa0/2 | 00E0.F705.6102 | - |
| Switch2 | Gig0/1 | 000C.CFBA.3C19 | - |
| Switch2 | Fa0/1 | 000C.CFBA.3C01 | - |
| Switch2 | Fa0/2 | 000C.CFBA.3C02 | - |
| PC0 | Fa0 | 000A.4136.39A4 | 192.168.137.2 |
| PC1 | Fa0 | 000C.85AB.1BA1 | 192.168.137.3 |
| PC2 | Fa0 | 0060.5C10.CEBD | 192.168.138.2 |
| PC3 | Fa0 | 000D.BD8A.04C7 | 192.168.138.3 |
| PC4 | Fa0 | 0060.5C5A.10A6 | 192.168.139.2 |
| PC5 | Fa0 | 00E0.A319.629A | 192.168.139.3 |

Список маршрутов

| Название устройства | IP-адрес сети назначения | Маска сети назначения | Тип и номер интерфейса для отправки | IP-адрес для отправки |
| --- | --- | --- | --- | --- |
| Router0 | 192.168.138.0 | 255.255.255.0 | Gig0/0 | 192.168.140.2 |
| Router0 | 192.168.139.0 | 255.255.255.0 | Gig0/0 | 192.168.140.2 |
| Router1 | 192.168.137.0 | 255.255.255.0 | Gig0/0 | 192.168.140.1 |
| Router1 | 192.168.139.0 | 255.255.255.0 | Gig0/2 | 192.168.141.2 |
| Router2 | 192.168.137.0 | 255.255.255.0 | Gig0/0 | 192.168.141.1 |
| Router2 | 192.168.138.0 | 255.255.255.0 | Gig0/0 | 192.168.141.1 |

Show IP route

Router0

Codes: L - local, C - connected, S - static, R - RIP, M - mobile, B - BGP

D - EIGRP, EX - EIGRP external, O - OSPF, IA - OSPF inter area

N1 - OSPF NSSA external type 1, N2 - OSPF NSSA external type 2

E1 - OSPF external type 1, E2 - OSPF external type 2, E - EGP

i - IS-IS, L1 - IS-IS level-1, L2 - IS-IS level-2, ia - IS-IS inter area

* - candidate default, U - per-user static route, o - ODR

P - periodic downloaded static route

Gateway of last resort is not set

192.168.137.0/24 is variably subnetted, 2 subnets, 2 masks

C 192.168.137.0/24 is directly connected, GigabitEthernet0/1

L 192.168.137.1/32 is directly connected, GigabitEthernet0/1

S 192.168.138.0/24 [1/0] via 192.168.140.2

S 192.168.139.0/24 [1/0] via 192.168.140.2

192.168.140.0/24 is variably subnetted, 2 subnets, 2 masks

C 192.168.140.0/24 is directly connected, GigabitEthernet0/0

L 192.168.140.1/32 is directly connected, GigabitEthernet0/0

Router1

Codes: L - local, C - connected, S - static, R - RIP, M - mobile, B - BGP

D - EIGRP, EX - EIGRP external, O - OSPF, IA - OSPF inter area

N1 - OSPF NSSA external type 1, N2 - OSPF NSSA external type 2

E1 - OSPF external type 1, E2 - OSPF external type 2, E - EGP

i - IS-IS, L1 - IS-IS level-1, L2 - IS-IS level-2, ia - IS-IS inter area

* - candidate default, U - per-user static route, o - ODR

P - periodic downloaded static route

Gateway of last resort is not set

S 192.168.137.0/24 [1/0] via 192.168.140.1

192.168.138.0/24 is variably subnetted, 2 subnets, 2 masks

C 192.168.138.0/24 is directly connected, GigabitEthernet0/1

L 192.168.138.1/32 is directly connected, GigabitEthernet0/1

S 192.168.139.0/24 [1/0] via 192.168.141.2

192.168.140.0/24 is variably subnetted, 2 subnets, 2 masks

C 192.168.140.0/24 is directly connected, GigabitEthernet0/0

L 192.168.140.2/32 is directly connected, GigabitEthernet0/0

192.168.141.0/24 is variably subnetted, 2 subnets, 2 masks

C 192.168.141.0/24 is directly connected, GigabitEthernet0/2

L 192.168.141.1/32 is directly connected, GigabitEthernet0/2

Router2

Codes: L - local, C - connected, S - static, R - RIP, M - mobile, B - BGP

D - EIGRP, EX - EIGRP external, O - OSPF, IA - OSPF inter area

N1 - OSPF NSSA external type 1, N2 - OSPF NSSA external type 2

E1 - OSPF external type 1, E2 - OSPF external type 2, E - EGP

i - IS-IS, L1 - IS-IS level-1, L2 - IS-IS level-2, ia - IS-IS inter area

* - candidate default, U - per-user static route, o - ODR

P - periodic downloaded static route

Gateway of last resort is not set

S 192.168.137.0/24 [1/0] via 192.168.141.1

S 192.168.138.0/24 [1/0] via 192.168.141.1

192.168.139.0/24 is variably subnetted, 2 subnets, 2 masks

C 192.168.139.0/24 is directly connected, GigabitEthernet0/1

L 192.168.139.1/32 is directly connected, GigabitEthernet0/1

192.168.141.0/24 is variably subnetted, 2 subnets, 2 masks

C 192.168.141.0/24 is directly connected, GigabitEthernet0/0

L 192.168.141.2/32 is directly connected, GigabitEthernet0/0